This CAD library is for CadSoft - EAGLE tool

How to import new library file (.lbr) into EAGLE:

1. Download and extract the file

2. Copy the library file (.lbr) to EAGLE library folder; "C:\Program Files\EAGLE-X.XX.X\lbr" (X.XX.X is your EAGLE software version)

3. Start EAGLE software

4. Under EAGLE Control Panel, go to the Library tree and browse for new copied library (.lbr) file

5. Right click on the library and select "USE"

6. You have successfully imported new library and ready to use in your design

How to search for a specific device in the EAGLE library tree

1. Navigate to the “View” menu under EAGLE Control Panel

2. Click on “search in tree”, the search bar will be shown at the bottom left of the Control panel

3. Enter the device name you are looking for and press enter

4. The result will be highlighted in the library tree

For further information on searching the EAGLE library tree, please consult the EAGLE help file

Note: The library file is NOT compatible with versions prior to 6.0